DISCLAIMER

Copyright 2025, Airworthiness Certification Services, LLC. (ACS).

All rights reserved.

These Templates are perpetually licensed for a single company, for the site and/or project team who initially purchased. The purchaser can use the Templates, or any customized variant of them, on as many projects as desired. The Templates may not be shared outside of the purchasing company with the exception of interactions with customers and program auditors. The Purchaser agrees to these terms as part of the sale. If a worldwide Corporate License is desired, which provides unlimited access for all sites and project teams within a company, contact info@avionics-certification-academy.com for details and a quote.

This Template is proprietary to ACS. and should not be distributed or copied in its original format without written permission. It is to be used solely as the basis of documentation for the purchaser of this template. Also, it should not be viewed as FAA opinion or regulatory guidance and presents only opinion and interpretation of FAA information by ACS.

All references to DO-178C and/or DO-254 material are copyrighted by and used with permission from:

RTCA, Inc.

1150 18th Street NW, Suite 910

Washington, DC 20036

(202) 833-9339

www.rtca.org

ACS hereby disclaims all warranties and conditions with respect to these Templates, either express, implied or statutory, including, but not limited to, the implied warranties and/or conditions of merchantability, of satisfactory quality, of fitness for a particular purpose, of accuracy, or quiet enjoyment, and non-infringement of third party rights. ACS does not warranty against interference with clients enjoyment of the Templates, that the functions contained in the Templates will meet the client’s requirements, that the Templates will be error-free or that defects in the templates will be corrected.

No oral or written information or advice given by ACS or by an ACS authorized representative shall create a warranty should the Templates prove defective, client assumes the entire cost of all necessary servicing, repair or correction.

Downloading and using these templates signifies you have read and accepted these conditions.

Please direct all questions or concerns to:

Airworthiness Certification Services
PMB#224, 5500 Olympic Drive, Suite H-105, Gig Harbor Wa 98335
Phone: 425-427-1956, email: info@airworthinesscert.com

NOTE: YOU MAY REMOVE THIS PAGE FROM YOUR TEMPLATES AFTER READING THIS DISCLAIMER

Tool Qualification Plan and Data

for the

 <Program Name>

Document No: <Doc Number>

Revision: -

| __________________________________________________ |  |  | ___________ |
| --- | --- | --- | --- |
| <Name>, Program Manager |  | Date |  |
| __________________________________________________ |  | ___________ |  |
| <Name>, Technical Project Lead |  | Date |  |
| __________________________________________________ |  | ___________ |  |
| <Name>, Engineer |  | Date |  |
| __________________________________________________ |  | ___________ |  |
| <Name>, Process Assurance Engineer |  | Date |  |

Notice This document and the information contained herein are the property of <Company Name>. Any reproduction, disclosure or use thereof is prohibited except as authorized in writing by <Company Name>. Recipient accepts the responsibility for maintaining the confidentiality of the contents of this document.

| REVISIONS |  |  |  |
| --- | --- | --- | --- |
| Rev. | Reason/Description | Requested/ Changed By | Date |

Table of Contents

Section 										            Page

1.0	INTRODUCTION	7

1.1	Purpose & Scope	7

1.2	Definitions & Acronyms	8

1.3	Applicable Documents	8

1.3.1	External Documents	8

1.3.2	Internal Documents	9

2.0	TOOL OVERVIEW	10

2.1	Configuration Identification	10

2.2	Tool Limitations	10

2.3	Tool Errata	10

3.0	CERTIFICATION CREDIT SOUGHT	11

4.0	TOOL OPERATIONAL REQUIREMENTS	12

4.1	Platform and Technical Requirements	12

4.2	Coding Standards Requirements	12

5.0	TOOL QUALIFICATION ACTIVITIES AND DATA	16

5.1	Tool Qualification Process and Activities	16

5.2	Tool Qualification Data	16

6.0	TEST CASE MAPPING	18

7.0	TOOL QUALIFICATION TESTING AND ANALYSIS RESULTS	19

8.0	ADDITIONAL CONSIDERATIONS	20

9.0	TOOL QUALIFICATION ACCOMPLISHMENT SUMMARY	21

List of Figures

No table of figures entries found.

List of Tables

Table 1-1. Definitions	8

Table 1-2. External Documents	8

Table 1-3. Internal Documents	9

Table 4-1. Table of TORs	12

Table 6-1. Test Case to TOR Mapping	18

Table 7-1. Test Results	19

# INTRODUCTION

Tool Qualification is described in DO-254 11.4. It’s a good idea to revisit that section before embarking on plans for tool qualification. While the topic is presented, and some guidance is provided within that section, what is actually required/accepted can vary greatly – depending on tool, its use, your verification strategy, and even your certification authority. In the software realm, DO-330 is the definitive standard for Tool Qualification. While it does not necessarily apply to DO-254/hardware, it may be of use to review that document as well as most certification authorities are familiar with it and may use the language and concepts presented there.

This template provides a basic example of what may be accepted to qualify a commonly used commercial verification tool – a Linting tool (used in a DAL A/B application with no manual checking of the coding rules for which the tool provides credit. For this example, we are going to use the real tool, Questa Lint, from the vendor Siemens. While this is a popular tool, it is not to be taken as an endorsement. It is to be considered an example only. However, if you plan to use this or another similar linting tool in a similar way, you may be able to reuse the strategy and much of this document’s content.

It is recommended that for any sort of “Design” tool that may need qualification (i.e., the results of the design tool are accepted without thorough verification) or with complex Verification tools for DAL A/B that are the sole means of verifying some aspects of a design, that you consult your certification authority and discuss acceptable strategies EARLY in the life cycle.

This document describes the plan for qualifying the recommended DO-254 rules of the ACME Questa Lint tool. The Questa Lint tool automatically checks VHDL or Verilog RTL code to ensure that it adheres to the proper hardware standards for coding. This qualification covers only the checks used in the <doc-ref>_HDS for this program.

The certification credit sought for the use of Questa Lint as a verification tool, is the reduction and automation of HDL code checking as described in AC/AMC 20-152A Section 5.10, as well as EASA Certification Memo SWCEH – 001 Issue: 01 Revision: 01 (3/9/12) section 8.4.2.1 f, as supplemental policy to RTCA/DO-254. The two sources of

The Questa Lint tool’s role as a code-checking tool puts it into the RTCA/DO-254 11.4 category of Verification tool and the TQL5 category as per RTCA/DO-330, “Software Tool Qualification Considerations.” While DO-330 is not officially mandated for DO-254 tool qualification, the tool qualification process outlined here supports both the DO-254 requirements as well as the DO-330 considerations.

The approach for this tool qualification is to limit the scope of usage of the tool to a predefined set of checks, demonstrate its proper use on a sample design, and then reuse this data in subsequent compliance projects where the usage of the tool can be shown to be identical.

## Purpose & Scope

The purpose of this document is to define the approach for the qualification of the Questa Lint tool for use within the scope of the <project-name> project. This tool is a verification tool, used in a DAL A/B application, and automates the objective of verifying that HDL code meets the defined design standards.

## Definitions & Acronyms

Copy pertinent definitions from the project PHAC and add any additional definitions (highlight in bold italics) that may have been missing at the time it was written.

The following table defines the key terms used in this project.

Table 1-1. Definitions

| Definition | Meaning |
| --- | --- |
| DR | Design Review (coding rule category) |
| CDC | Clock Domain Crossing |
| CP | Coding Practices (coding rule category) |
| HDL | Hardware Description Language |
| RTL | Register Transfer Level |
| SS | Safe Synthesis (coding rule category) |
| TOR | Tool Operational Requirements |
| TQL | Tool Qualification Level |
| VHDL | VHSIC (Very High-Speed Integrated Circuit) Hardware Description Language |

## Applicable Documents

List all the documents and their revisions that are either referenced directly or have some bearing on this Tool Qualification planning document.

The following documents are applicable to this project.

### External Documents

Update the list below from what was initially provided in the PHAC where needed.

The following external documents have a bearing on the certification requirements of this project.

Table 1-2. External Documents

| Document Number | Title |
| --- | --- |
| RTCA/DO-254 | Design Assurance for Airborne Electronic Hardware |
| FAA Order 8110.4C | Type Certification |
| FAA Order 8110.105B | FAA, Simple and Complex Electronic Hardware Approval Guidance |
| CM-SWCEH-001 Issue 01 Revision 02 | Certification Memorandum: Development Assurance of Airborne Electronic Hardware |
| AC/AMC 20-152A | (FAA/EASA) Development Assurance for Airborne Electronic Hardware (AEH) |
| RTCA/DO-330 | Software Tool Qualification Considerations |
| DO254-UG-001 | Best Practice VHDL Coding Standards for DO-254 Programs (Rev 1a) |
| <Siemens Doc Ref> | Questa Lint User and Reference Manual |

### Internal Documents

The following internal <company-name> documents contain additional detail about the DO-254 compliance aspects of this program that may relate to the use of this tool and its tool qualification.

Table 1-3. Internal Documents

| Document Number | Revision | Title | Certification Requirement |
| --- | --- | --- | --- |
| <doc-ref#>_PHAC | 0.1 | Plan for Hardware Aspects of Certification | DO-254 10.1.1 |
| <doc-ref#>_HDP | FILL IN | Hardware Design Plan | DO-254 10.2 |
| <doc-ref#>_HVVP |  | Hardware Validation & Verification Plan | DO-254 10.1.3/10.1.4 |
| <doc-ref#>_HCMP |  | Configuration Management Plan | DO-254 10.1.5 |
| <doc-ref#>_HPAP |  | Process Assurance Plan | DO-254 10.1.6 |
| <doc-ref#>_HDS |  | Hardware Design Standards | DO-254 10.2.2 |
| <doc-ref#>_HVVS |  | Hardware Validation and Verification Standards | DO-254 10.2.3 |
| <doc-ref#>_HVR |  | Hardware Verification Results | DO-254 10.4.5 |
| <doc-ref#>_HCI-HECI |  | Hardware Configuration Index; Hardware Environment Configuration Index | DO254 10.3.2.2.1; AC/AMC 20-152A 5.10 |
| <doc-ref#>_VCI |  | Hardware Verification Index | Best Practice |
| <doc-ref#>_HAS |  | Hardware Accomplishment Summary | DO-254 10.9 |
| <doc-ref#>_HW-Records |  | Hardware Records | DO-254 10.6-8 |

# TOOL OVERVIEW

Questa Lint is used to both provide a fast check of this project’s HDL code (prior to testbench development), as well as to score the code for completeness and consistency issues, using syntactic, semantic, stylistic, and structural analyses. In this usage, Questa Lint identifies issues when introduced, preventing issues from becoming more expensive to fix later. Much of what Questa Lint does at this early verification stage is verified again with the project’s official testbench and simulation methods. However, in terms of verification automation that isn’t reverified (and thus must be “qualified”), Questa Lint automatically verifies if the HDL code (used as tool input) meets the “rules” (another tool input) that have been defined for that code. As output, Questa Lint produces rules violations, including metrics and quality scores for the code.

In the typical usage model, the user selects the coding rules they want to run (in this case, the set defined as the “DO-254 Rules”), identifies the HDL code file they want to check, and then runs Questa Lint to check the file’s HDL code against the specified coding rules. At the conclusion of the process, the tool outputs a report highlighting which rules passed and which failed. The user can set how much detail they want to see in the report.

Specific to its use in this DO-254 program, the automation provided by the tool eliminates manual checking of defined HDL design coding standards.

## Configuration Identification

This document is applicable to Questa Lint, a software tool that can be identified by Siemens <part number XXXX>. The version being used for this Tool Qualification is 10.4.c.

In the vendor (Siemens) environment, the development process is tightly controlled under proprietary processes for Planning, Requirements Capture, Design, Verification, Configuration Management (including defect tracking and management, and change control) and Release. If more information is required on these processes, contact Siemens Questa Support Team.

In the project environment, the Questa Lint tool is kept under configuration management as per the <doc-ref>_HCMP.

## Tool Limitations

Questa Lint performs automated checking of a superset of rules recommended by Siemens for DO-254 compliant designs. This set of rules includes those in the DO-254 User Group’s position paper on HDL code checking.

The tool works on any VHDL, Verilog or SystemVerilog code. Not all rules are appropriate for each HDL coding language. This program will use rules pertinent to VHDL only.

## Tool Errata

Note that you need to verify this prior to running Tool Qualification for credit.

No known defects against the DO-254 coding rules to be used in this program are open against version 10.4.c.

# CERTIFICATION CREDIT SOUGHT

The program team using Questa Lint for DO-254 rule checking is seeking certification credit for the reduction and automation of manual review of HDL coding standards, which is considered a verification activity. While DO-254 mentions hardware design standards in various places, it is AC/AMC 20-152A and EASA CM-SWCEH-001 Issue 01 Revision 02 that clearly describe the need for verifying that HDL code complies with pertinent coding standards.

For this qualification submittal, the configuration of the tool with respect to both the design environment and operational requirements are specified in Section 4.0 “TOOL OPERATIONAL REQUIREMENTS”. This qualification will be used to establish a baseline from which change impact and regression analysis can be performed.

The means of compliance will be to clearly define the standards that the tool will be checking the HDL against as "rules”. These are described as the Tool Operational Requirements. A test will be developed that consists of one or more designs, coded in the same language with the same constraints as the project, which violate each rule at least once. The expected results (as determined by manual review) will be captured and compared against the tool results, and any discrepancies will be evaluated.

This is described in more detail in Section 5.0 “TOOL QUALIFICATION ACTIVITIES AND DATA”.

# TOOL OPERATIONAL REQUIREMENTS

## Platform and Technical Requirements

Questa Lint supports (<define acceptable HW platform>). In this program, designers will be using Dell Inspiron PCs running Windows 11. This hardware platform is supported by the Questa Lint 10.4.c release. Tool installation testing will be run to ensure the tool is set up properly. This test will be part of the verification test bench.

## Coding Standards Requirements

Note that if you have a more extensive standards/rule set that you are checking with your tool, ensure you add the additional standards/rules to this description.

The following table defines the Questa Lint tool operational requirements (TOR). Credit is sought for the automation of DO-254 rules checking to meet the VHDL coding standards defined in this program (see <doc-ref>_HDS). The TOR therefore consists of each of these coding rules. The table maps a category of rules, to the DO-254 User Group paper Rule ID, and then maps this to the Questa Lint rule ID, and describes the intent of the rule.

Table 4-1. Table of TORs

| Category | DO-254 UG Rule ID: Name | Questa Lint Rule ID | Description |
| --- | --- | --- | --- |
| Coding Practices | CP1: Avoid Incorrect VHDL Type Usage | QL-XXX | Check for the incorrect use of types, incompatible bounds, and/or constraints. |
|  | CP2: Avoid Duplicate Signal Assignments |  | The same signal should not be assigned a value more than once within the same statement region. |
|  | CP3: Avoid Hard-Coded Numeric Values |  | For design IP reuse and portability ease, hard-coded numeric values should not be used. |
|  | CP4: Avoid Hard-Coded Vector Assignment |  | For vector reset assignments; do not use hard-coded values. |
|  | CP5: Ensure Consistent FSM State Encoding Style |  | a. A design should employ a consistent state encoding style for Finite State Machines (FSM). b. FSM state types should not be hard-coded, unless unavoidable. |
|  | CP6: Ensure Safe FSM Transitions |  | a. All unused (illegal or undefined) states should transition to a defined recovery state, where this error condition can be processed accordingly. b. There should be no unreachable states (i.e., those without any incoming transitions) and dead-end states (i.e., those without any outgoing transitions) in a FSM. |
|  | CP7: Avoid Mismatching Ranges |  | Bit widths on both sides of an assignment, comparison, or association should match. |
|  | CP8: Ensure Complete Sensitivity List |  | The sensitivity list should only contain the signals needed by the process. |
|  | CP9: Ensure Proper Sub-Program Body |  | Each sub-program must: a. have only one exit point b. have no recursion c. access only local variables/signals |
|  | CP10: Assign Value Before Using |  | Every object (e.g. signal, variable, port) should be assigned a value before using it. |
|  | CP11: Avoid Unconnected Input Ports |  | All input ports should be driven. |
|  | CP12: Avoid Unconnected Output Ports |  | Design output ports should be connected. |
|  | CP13: Declare Objects Before Use |  | Objects should be declared before use. |
|  | CP14: Avoid Unused Declarations |  | All declared objects should be used. |
| Safe Synthesis | SS1: Avoid Implied Logic |  | Do not allow coding that implies feed-throughs, delay chains, and internal tri-state drivers. |
|  | SS2: Ensure Proper Case Statement Specification |  | Case statements should: a. Be complete b. Never have duplicate/overlapping statements c. Never have unreachable case items d. Always include the “when others” clause |
|  | SS3: Avoid Combinational Feedback |  | Do not allow combinational feedback paths. |
|  | SS4: Avoid Latch Inference |  | The HDL coding style should avoid inference of latches. |
|  | SS5: Avoid Multiple Waveforms |  | Only one waveform should exist on the right-hand side of a signal assignment. |
|  | SS6: Avoid Multiple Drivers |  | The same signal/variable should be assigned in only one sequential block. |
|  | SS7: Avoid Uninitialized VHDL Deferred Constants |  | Ensure all VHDL deferred constants are initialized. |
|  | SS8: Avoid Clock Used as Data |  | Clock signals should not be used in a logic path that drives the data input of a register. |
|  | SS9: Avoid Shared Clock and Reset Signal |  | The same signal should not be used as both a clock and reset signal. |
|  | SS10: Avoid Gated Clocks |  | Data signals should not be used in a logic path that drives the clock input of a register. |
|  | SS11: Avoid Internally Generated Clocks |  | Internally generated clocks should be avoided. |
|  | SS12: Avoid Internally Generated Resets |  | Unless they are properly isolated, internally-generated resets should be avoided. |
|  | SS13: Avoid Mixed Polarity Reset |  | The same reset signal should not be used with mixed styles or polarities. |
|  | SS14: Avoid Non-resettable Registers |  | All registers should have a reset control. |
|  | SS15: Avoid Asynchronous Reset Release |  | Reset signals should have a synchronous release. |
|  | SS16: Avoid Initialization Assignments |  | Do not use register initialization assignments. |
|  | SS17: Avoid Un-driven and Unused Logic |  | a. Every register and latch must be used and driven in the design. b. Registers and latches affecting only unused logic must be examined. |
|  | SS18: Ensure Register Controllability |  | Each register should be controllable from its inputs. |
|  | SS19: Avoid Snake Paths |  | Combinational paths should not exceed a maximum allowable logic depth. |
|  | SS20: Ensure Nesting Limits |  | Conditional branching constructs should have a maximum nesting depth. |
|  | SS21: Ensure Consistent Vector Order |  | Use the same multi-bit vector order consistently throughout the design. |
| Design Reviews | DR1: Use Statement Labels |  | Statements should have labels. |
|  | DR2: Avoid Mixed Case Naming for Differentiation |  | Names should not be differentiated by case alone. |
|  | DR3: Ensure Unique Name Spaces |  | The same name should not be used for different types of identifiers. |
|  | DR4: Use Separate Declaration Style |  | Each declaration should be placed on a separate line. |
|  | DR5: Use Separate Statement Style |  | Each statement should be placed on a separate line. |
|  | DR6: Ensure Consistent Indentation |  | Code should be consistently indented. |
|  | DR7: Avoid Using Tabs |  | Tabs should not be used. |
|  | DR8: Avoid Large Design Files |  | Designs should be partitioned and files should be of limited size. |
|  | DR10: Ensure Consistent File Header |  | File headers should be consistent. |
|  | DR11: Ensure Sufficient Comment Density |  | Code should be sufficiently documented via inline comments. |
|  | DR12: Ensure Proper Placement of Comments |  | Comments should be placed in appropriate places to aid understanding. |

# TOOL QUALIFICATION ACTIVITIES AND DATA

The tool qualification activities, as well as the data produced are described in the following subsections.

## Tool Qualification Process and Activities

The tool qualification activities for the Questa Lint HDL DO-254 Rule Checking feature, includes several key parts as describe here. The assigned team member (<define who this will be>) will perform the following:

- Contact Siemens Questa support for updated information on release 10.4.c. Evaluate any defects that may affect the planned usage/features of the tool, and document any relevant issues in Section 2.3, “Tool Errata.”

- Confirm with the design team, both the features requiring qualification in Questa Lint, along with the platform and technical requirements on which the tool is running. These features are documented in Section 4.0, “TOOL OPERATIONAL REQUIREMENTS.”

- Run on site, in the specified environment, a set of appropriate feature-based tests that map to the DO-254 coding standards/rules being used on the project. Note that this set of tests includes testing for proper tool installation.

- Create a file containing this information and name it “Questa Lint-ToolQual-TestMapping.xls.” 
Note that the test mapping file is just the TOR with extra columns to describe the tests associated with each requirement (i.e. HDL coding rule).

- Ensure these tests are documented and map to the rules they test. Document the test mapping in Section 6.0 of this document.

- Ensure the tests to verify the tool’s operational requirements reside in a directory called “Questa Lint_Tests.”

- Execute the tests by running a script called “Questa Lint_RunScript.bat” in the Questa Lint_Tests folder. This script launches the tool, executes the tests, and captures test results.

- Examine the “Questa Lint-ToolQual-Log.csv” file, which includes detail about whether the tests passed or failed for each rule, and analyzes actual versus expected results, to verify that the tool performed as expected.

- Examine, troubleshoot, and document any unexpected results.

- Document actual results in Section 7.0, “TOOL QUALIFICATION TESTING AND ANALYSIS RESULTSL OPERATIONAL REQUIREMENTS,” of this document.

## Tool Qualification Data

This entire document will be made available to the certification authority. The sections of most interest include:

- Sections 1.0, 2.0, 3.0 and 5.0, which serve as The Tool Qualification Plan. This document, and/or these sections, should be referenced in the PHAC.

- The Tool Operational Requirements are documented in Section 4.0.

- The test case procedures and mapping of test cases to the TOR is described in Section 6.0.

- The test procedures are defined in “QL-ToolQual-TestProcedures.doc” and the actual tests are in the “QL_Tests” folder. 
(Note that we have not provided the actual test sets, scripts, logs or the fictitious documents/folder referenced above or below. These are provided just to give you an idea of how you can set this up and what documentation may be needed).

- The tests are run by executing “QL_RunScript.bat” in the QL_Tests directory.

- Section 7.0 documents the Tool Qualification Results.

- Section 9.0 includes a summary of the Tool Qualification Accomplishment Summary.

# TEST CASE MAPPING

In this section, include a mapping of each test case to the rule(s) (from the TOR) that it verifies. Also clarify where these tests are located and how they are run.

The tests for this qualification reside in <path>/QL_Tests folder within the <describe environment> development environment and will be run with the QL_RunScript.bat script.

The following table links the tests that will be run to their corresponding TOR.

Table 6-1. Test Case to TOR Mapping

| Test or Questa Lint Rule ID | Test Case Directory/Test Name | Expected Results |
| --- | --- | --- |
| Tool Install | <path>/QL_Tests/<test_name> | Pass |
| QL-xxx | <path>/QL_Tests/<test_name> | Pass |
| QL-yyy | <path>/QL_Tests/<test_name> | Fail |
| … |  |  |

# TOOL QUALIFICATION TESTING AND ANALYSIS RESULTS

The Tool Qualification process was run just as described in this document. The test engineer ran the tests (provide some details here – who, when, where, how) and verified the results.

The details for the run were captured in the <path>/QL_test_run.log file. The test results are captured in summary form in the table below. There were no unexpected results and therefore no issues to justify. The tool performed as expected.

Table 7-1. Test Results

| Test or Questa Lint Rule ID | Test Case Directory/Test Name | Expected Results | Actual Results | Manually Verified |
| --- | --- | --- | --- | --- |
| Tool Install | <path>/QL_Tests/<test_name> | Pass | Pass | X |
| QL-xxx | <path>/QL_Tests/<test_name> | Pass | Pass | X |
| QL-yyy | <path>/QL_Tests/<test_name> | Fail | Fail | X |
| … |  |  |  |  |

# ADDITIONAL CONSIDERATIONS

DO-330 Section 11 introduces some additional considerations. These considerations, and their pertinence to this qualification, are described here:

- Multi-Function Tools
While Questa Lint itself is a multi-function tool, only the specific function of HDL code checking for the recommended DO-254 compliance rules is addressed with this qualification. Any modifications to the ruleset will require modifications to the test set needed to qualify the feature. Any other features of the Questa Lint tool may need their own qualification process.

- Previously Qualified Tools
This qualification process and results are pertinent for all uses of the same version of the tool in the same operational environment. Changes to the tool version or operational environment necessitate re-running and possible modification to the tests applied. Multiple applicant projects may reference and reuse the results of this qualification as long as the full environment specified remains the same. Changes in the TOR, tool version, operating environment, etc. will require a requalification.

- Qualifying COTS Tools
Questa Lint is a COTS tool. DO-330 guidance for COTS tools of level TQL-5 were followed in this qualification process.

- Service History
Service history is not necessary for TQL-5 COTS tool qualification. Likewise, TQL-5 COTS tools do not require data from the Tool Developer.

- Alternative Methods for Tool Qualification
No alternative methods of tool qualification were used. DO-330 guidance for COTS tools of level TQL-5 were followed in this qualification process.

# TOOL QUALIFICATION ACCOMPLISHMENT SUMMARY

Here you’ll include a high-level statement of how/why the tool qualification process performed complies with DO-254, the results and that the tool is now qualified.

This document summarizes the plan, process, data, and results for Tool Qualification of the Questa Lint tool. The document has been prepared in accordance with RTCA/DO-254 Section 11.4, with considerations for compliance to RTCA/DO-330 as well.

The approach presented earlier in this document is compliant with DO-254 and DO-330, and adheres to the tool usage as defined in the hardware plans.

The steps were performed as documented, and the results captured and reviewed. The results indicate the tool performs its TOR as expected.

No additional considerations were required in this tool qualification.

The Certification Liaison reviewed this data as part of the SOI-1 (Plan) and SOI-3a (Results) reviews. There were no findings.

Therefore, the Questa Lint tool is considered Qualified in the context of this program.